📡

CAN BUS PROJECT

Simulation of CAN Protocol

Technical Documentation

| Platform | Arduino UNO (×2) |
| --- | --- |
| CAN Controller | MCP2515 Module |
| Interface | SPI |
| Protocol | CAN 2.0A / 2.0B |
| Language | C / C++ (Arduino) |
| Baud Rate | 500 kbps |

Automotive Communication  ·  Embedded Systems  ·  Real-Time Data Transfer

# 1. Project Overview

This project implements a hardware-level simulation of the Controller Area Network (CAN) protocol — the backbone communication standard used in virtually every modern vehicle. Two Arduino UNO microcontrollers communicate via MCP2515 CAN controller modules, exchanging real-time temperature and humidity sensor data across a CAN bus network.

## 1.1  Motivation

CAN bus is used in every modern vehicle's Electronic Control Unit (ECU) network — from engine management and ABS to infotainment and body electronics. Understanding CAN protocol at the hardware level is directly relevant to careers in automotive embedded systems, robotics, and industrial automation.

## 1.2  Key Achievements

- Implemented full CAN communication between two Arduino UNO boards

- Transmitted real-time sensor data (temperature & humidity) over CAN

- Demonstrated message arbitration and priority-based bus access

- Achieved reliable data transfer with error detection

- Simulated a multi-node ECU-like network topology

📌 Why CAN Bus? CAN (ISO 11898) is a multi-master, multi-drop serial bus protocol designed for robust communication in electrically noisy environments. Every modern vehicle uses CAN to let dozens of ECUs share data — braking, steering, engine, airbags — without point-to-point wiring between each unit.

# 2. System Architecture

## 2.1  Block Diagram

The system consists of two Arduino UNO nodes connected via the MCP2515 CAN controller. Each node communicates with its MCP2515 over SPI, and the two MCP2515 modules are wired together via the CAN_H and CAN_L differential pair. A 120 Ω termination resistor is placed at each end of the bus.

[Node A — Transmitter] [Node B — Receiver]

┌─────────────────────┐ ┌────────────────────┐

│ Arduino UNO │ │ Arduino UNO │

│ ┌──────────────┐ │ CAN_H │ ┌─────────────┐ │

│ │ MCP2515 │──────────────────│ │ MCP2515 │ │

│ │ CAN Ctrl │──────────────────│ │ CAN Ctrl │ │

│ └──────────────┘ │ CAN_L │ └─────────────┘ │

│ │ DHT22 Sensor│ │ │ │ Serial Monitor│ │

└─────────────────────┘ └────────────────────┘

(SPI Interface) (SPI Interface)

120Ω ─┤ ├─ 120Ω

## 2.2  Communication Flow

- Node A reads temperature and humidity from the DHT22 sensor

- Data is packed into a CAN frame (11-bit standard ID or 29-bit extended ID)

- MCP2515 transmits the frame onto the CAN bus differential pair

- Node B's MCP2515 receives the frame and raises an interrupt

- Node B unpacks the data and outputs it to Serial Monitor

## 2.3  Protocol Specifications

| Parameter | Value / Detail |
| --- | --- |
| CAN Standard | ISO 11898-1 (CAN 2.0A / 2.0B) |
| Bus Speed | 500 kbps |
| Frame Type | Standard (11-bit CAN ID) |
| Message ID (Temp) | 0x100 |
| Message ID (Humidity) | 0x101 |
| Data Length Code | 8 bytes per frame |
| Termination | 120 Ω resistors at both bus ends |
| Controller Interface | SPI at 10 MHz |
| MCP2515 Oscillator | 8 MHz crystal |

# 3. Hardware Components

## 3.1  Bill of Materials

| Parameter | Value / Detail |
| --- | --- |
| Arduino UNO R3 (×2) | ATmega328P microcontroller, 5V logic, SPI hardware support |
| MCP2515 CAN Module (×2) | Microchip MCP2515 CAN controller with TJA1050 transceiver, SPI interface |
| DHT22 Sensor (×1) | Digital temperature & humidity sensor, ±0.5°C accuracy |
| 120 Ω Resistors (×2) | CAN bus termination resistors (placed at each node) |
| Breadboard & Jumper Wires | For prototyping and connections |
| USB Cables (×2) | Power and serial communication to PC |

## 3.2  MCP2515 — CAN Controller

The MCP2515 is a stand-alone CAN controller that interfaces with the Arduino via SPI. It implements the full CAN 2.0A/2.0B protocol, handling frame assembly, bit timing, arbitration, error detection, and acknowledgment autonomously.

### 3.2.1  Key Features

- Full CAN 2.0A/2.0B protocol implementation

- 3 transmit and 2 receive buffers

- 6 acceptance filters and 2 acceptance masks for hardware filtering

- Interrupt-driven operation via INT pin

- SPI interface up to 10 MHz

- Integrated TJA1050 transceiver on module for differential signaling

## 3.3  Pin Connections

### Node A (Transmitter) — Arduino UNO ↔ MCP2515

| Pin / Signal | Arduino UNO | MCP2515 Module | Description |
| --- | --- | --- | --- |
| VCC | 5V | VCC | Power supply |
| GND | GND | GND | Common ground |
| CS (Chip Select) | Pin 10 (SS) | CS | SPI chip select — active LOW |
| SCK (Clock) | Pin 13 (SCK) | SCK | SPI clock signal |
| SI / MOSI | Pin 11 (MOSI) | SI | SPI data — Master Out |
| SO / MISO | Pin 12 (MISO) | SO | SPI data — Master In |
| INT (Interrupt) | Pin 2 (INT0) | INT | Active-LOW interrupt from MCP2515 |
| DHT22 Data | Pin 7 | — | Temperature/humidity sensor data |

### Node B (Receiver) — Arduino UNO ↔ MCP2515

| Pin / Signal | Arduino UNO | MCP2515 Module | Description |
| --- | --- | --- | --- |
| VCC | 5V | VCC | Power supply |
| GND | GND | GND | Common ground |
| CS (Chip Select) | Pin 10 (SS) | CS | SPI chip select |
| SCK (Clock) | Pin 13 (SCK) | SCK | SPI clock |
| SI / MOSI | Pin 11 (MOSI) | SI | SPI data — Master Out |
| SO / MISO | Pin 12 (MISO) | SO | SPI data — Master In |
| INT (Interrupt) | Pin 2 (INT0) | INT | Receive interrupt trigger |

# 4. CAN Protocol Deep Dive

## 4.1  CAN Frame Structure

A standard CAN 2.0A frame consists of the following fields. Understanding each field is key to implementing correct CAN communication.

┌─────┬──────────┬─────┬─────┬──────────────────┬─────┬─────┐

│ SOF │ CAN ID │ RTR │ DLC │ Data (0–8B) │ CRC │ ACK │

│ 1b │ 11b │ 1b │ 4b │ 0–64 bits │ 16b │ 2b │

└─────┴──────────┴─────┴─────┴──────────────────┴─────┴─────┘

SOF = Start of Frame RTR = Remote Transmission Request

DLC = Data Length Code CRC = Cyclic Redundancy Check

## 4.2  Message Priority & Arbitration

CAN bus implements non-destructive bitwise arbitration. When two nodes transmit simultaneously, the node with the lower CAN ID wins bus access — lower ID = higher priority. Losing nodes automatically retry after the winning message completes. This project assigns:

- 0x100  —  Temperature data (lower ID = higher priority)

- 0x101  —  Humidity data

💡 Arbitration Mechanism CAN uses dominant (logic 0) and recessive (logic 1) bit states. During arbitration, a node transmitting a recessive bit but detecting a dominant bit on the bus knows it has lost arbitration and immediately stops transmitting. The winning node never knows contention occurred.

## 4.3  Error Detection

CAN has multiple built-in error detection mechanisms, making it extremely robust for automotive environments:

- CRC Check — 15-bit CRC detects burst errors up to 5 bits

- Bit Monitoring — Every transmitter monitors what it sends

- Bit Stuffing — Prevents long runs of same bit polarity

- Frame Check — Verifies fixed-format fields

- ACK Check — Confirms at least one receiver acknowledged

## 4.4  Differential Signaling (CAN_H / CAN_L)

CAN bus uses differential voltage signaling for noise immunity. The TJA1050 transceiver on the MCP2515 module handles this conversion between TTL logic and differential bus levels:

State CAN_H (V) CAN_L (V) Differential

──────────────────────────────────────────────────

Recessive 2.5 V 2.5 V ~0 V

Dominant 3.5 V 1.5 V ~2 V

# 5. Firmware Implementation

## 5.1  Libraries Used

- mcp_can.h

- Open-source MCP2515 library by Cory J. Fowler

- Provides: begin(), sendMsgBuf(), readMsgBuf(), checkReceive()

- SPI.h

- Arduino built-in SPI library for hardware SPI communication

- DHT.h

- DHT sensor library for reading temperature and humidity

## 5.2  Node A — Transmitter Code

#include <SPI.h>

#include <mcp_can.h>

#include <DHT.h>

#define CAN_CS_PIN 10

#define DHT_PIN 7

#define DHT_TYPE DHT22

MCP_CAN CAN(CAN_CS_PIN);

DHT dht(DHT_PIN, DHT_TYPE);

// CAN Message IDs

const unsigned long TEMP_ID = 0x100;

const unsigned long HUMIDITY_ID = 0x101;

void setup() {

Serial.begin(115200);

dht.begin();

// Initialize MCP2515 at 500 kbps with 8 MHz oscillator

while (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN BUS init failed. Retrying...");

delay(1000);

}

CAN.setMode(MCP_NORMAL);

Serial.println("CAN BUS Initialized — Node A (Transmitter)");

}

void loop() {

float temperature = dht.readTemperature(); // Celsius

float humidity = dht.readHumidity(); // %RH

if (isnan(temperature) ¦¦ isnan(humidity)) {

Serial.println("DHT22 read error");

delay(2000);

return;

}

// Pack float into 4-byte array via union

byte tempBuf[8] = {0};

memcpy(tempBuf, &temperature, sizeof(float));

CAN.sendMsgBuf(TEMP_ID, 0, 4, tempBuf);

Serial.print("TX Temp: "); Serial.println(temperature);

byte humBuf[8] = {0};

memcpy(humBuf, &humidity, sizeof(float));

CAN.sendMsgBuf(HUMIDITY_ID, 0, 4, humBuf);

Serial.print("TX Humidity: "); Serial.println(humidity);

delay(1000); // Transmit every 1 second

}

## 5.3  Node B — Receiver Code

#include <SPI.h>

#include <mcp_can.h>

#define CAN_CS_PIN 10

#define CAN_INT_PIN 2

MCP_CAN CAN(CAN_CS_PIN);

void setup() {

Serial.begin(115200);

pinMode(CAN_INT_PIN, INPUT);

while (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) != CAN_OK) {

Serial.println("CAN BUS init failed. Retrying...");

delay(1000);

}

CAN.setMode(MCP_NORMAL);

Serial.println("CAN BUS Initialized — Node B (Receiver)");

}

void loop() {

// Poll INT pin — LOW when message available

if (!digitalRead(CAN_INT_PIN)) {

unsigned long rxId;

byte dlc;

byte rxBuf[8];

CAN.readMsgBuf(&rxId, &dlc, rxBuf);

float value;

memcpy(&value, rxBuf, sizeof(float));

if (rxId == 0x100) {

Serial.print("RX Temperature : ");

Serial.print(value); Serial.println(" C");

} else if (rxId == 0x101) {

Serial.print("RX Humidity : ");

Serial.print(value); Serial.println(" %");

}

}

}

# 6. Testing & Results

## 6.1  Test Setup

Both Arduino UNO boards were powered via USB and connected to the same PC for serial monitoring. The CAN bus was terminated with 120 Ω resistors at both ends. Tests were conducted at room temperature with ambient humidity readings.

## 6.2  Serial Monitor Output — Node A (TX)

CAN BUS Initialized — Node A (Transmitter)

TX Temp: 24.50

TX Humidity: 58.30

TX Temp: 24.52

TX Humidity: 58.28

TX Temp: 24.51

TX Humidity: 58.31

## 6.3  Serial Monitor Output — Node B (RX)

CAN BUS Initialized — Node B (Receiver)

RX Temperature : 24.50 C

RX Humidity : 58.30 %

RX Temperature : 24.52 C

RX Humidity : 58.28 %

RX Temperature : 24.51 C

RX Humidity : 58.31 %

## 6.4  Test Results Summary

| Parameter | Value / Detail |
| --- | --- |
| Bus Initialization | ✅ Successful — both nodes detected at 500 kbps |
| Message Transmission | ✅ 1 Hz update rate — no dropped frames observed |
| Message Reception | ✅ All frames received correctly via INT pin polling |
| Data Integrity | ✅ Float data reconstructed accurately via memcpy |
| Arbitration Test | ✅ Simultaneous TX — 0x100 consistently won priority |
| Error Handling | ✅ MCP2515 auto-retransmit on bus error |
| Termination | ✅ Signal integrity confirmed — no reflections observed |

# 7. Real-World Relevance

## 7.1  CAN Bus in Automotive ECUs

Modern vehicles contain 50–100 Electronic Control Units (ECUs) that communicate via CAN bus. Each ECU is a node — exactly like the Arduino UNOs in this project — that transmits and receives data on a shared bus. Examples include:

- Engine Control Module (ECM) — RPM, throttle, fuel injection timing

- Anti-lock Braking System (ABS) — Wheel speed data to chassis controller

- Airbag Control Unit — Collision detection, deployment signals

- Body Control Module (BCM) — Lighting, windows, door locks

- Transmission Control Module — Gear position, shift commands

## 7.2  Project Skills Mapped to Industry

| Parameter | Value / Detail |
| --- | --- |
| SPI bus communication | Used in every automotive-grade microcontroller interface |
| Interrupt-driven CAN RX | Standard pattern in AUTOSAR-based ECU software |
| Message ID / priority design | Core skill in CAN database (DBC) development |
| Sensor data packing | Signal encoding in Vector CANdb++ / PEAK tools |
| Multi-node bus topology | Directly mirrors OBD-II / vehicle CAN networks |
| Error detection awareness | Foundation for ISO 26262 functional safety |

🏁 Industry Connection This simulation mirrors what happens inside a real vehicle at a protocol level. The MCP2515 uses the same CAN 2.0B specification as automotive-grade controllers from NXP, Infineon, and Texas Instruments. Skills developed here — message framing, arbitration, interrupt handling, and SPI configuration — are directly transferable to industry tools like Vector CANalyzer, Kvaser, and PEAK USB-CAN interfaces.

# 8. Future Extensions

## 8.1  Planned Enhancements

- Add a third node to simulate multi-node arbitration

- Implement CAN FD (Flexible Data-rate) for 8 Mbps payloads

- Decode frames using a USB-CAN adapter and Vector CANalyzer

- Build a DBC (CAN database) file for signal decoding

- Add OLED display on Node B for real-time data visualization

- Implement CAN filtering and masking for selective reception

## 8.2  Migration Path

To scale this to automotive-grade hardware, the next step would be using STM32 or NXP S32K MCUs with built-in CAN peripherals, removing the need for an external MCP2515 controller. The software architecture — message packing, ID assignment, interrupt handling — remains identical.

# 9. References & Resources

- Microchip MCP2515 Datasheet — DS20001801H

- ISO 11898-1:2015 — Road vehicles — CAN protocol specification

- Arduino SPI Library Documentation — arduino.cc/reference/en/language/functions/communication/spi

- mcp_can Library — github.com/coryjfowler/MCP_CAN_lib

- CAN Bus Explained — CSS Electronics (csselectronics.com)

- AUTOSAR CAN Driver Specification — autosar.org